OPC-UA to MQTT Bridge

Technical Documentation

IoT Academy — Conestoga College

Project: OPC_UA-MQTT  |  Workstation: iot-group1

Date: June 10, 2026

1. Project Overview

OPC_UA-MQTT is an industrial IoT bridge application written in TypeScript (Node.js). It continuously reads live process data from a PLC via the OPC-UA protocol and republishes that data to an MQTT broker using a Unified Namespace (UNS) topic hierarchy. This makes real-time machine state available to dashboards, analytics pipelines, and other MQTT consumers without exposing the OPC-UA server directly.

Key Technologies

| Technology | Version / Library | Role |
| --- | --- | --- |
| TypeScript / Node.js | node-opcua-client | Runtime and OPC-UA client library |
| MQTT | mqtt.js | Publish process data to broker |
| OPC-UA | IEC 62541 | Read tag values from PLC |
| UNS | ISA-95 hierarchy | Structured MQTT topic naming |

Design Goals

- Publish related tags atomically — all fields in a group come from the same polling instant.

- Recover automatically from OPC-UA and MQTT outages without operator intervention.

- Never allow polling cycles to stack up, even under a slow or unresponsive server.

- Terminate cleanly when stopped, avoiding orphaned sessions on the OPC-UA server.

2. System Architecture

The bridge operates as a single-process Node.js application with two persistent connections: one to the OPC-UA server on the PLC network and one to the MQTT broker. A recursive timer drives the polling loop, reading all tag groups in sequence and publishing each group as a JSON message.

Data Flow

PLC (OPC-UA Server)

|

|  opc.tcp://192.168.232.30:26543

|

[ node-opcua client ]  <-- connection_lost / connection_reestablished events

|

readGroup()  -->  TAG_GROUPS definition

|

publishGroup()  -->  JSON payload  { timestamp, field1, field2, ... }

|

[ mqtt.js client ]  <-- auto-reconnect

|

|  mqtt://192.168.232.31:1883

|

MQTT Broker  -->  dashboards / analytics / historians

Polling Loop (Overlap-Safe)

A recursive setTimeout pattern is used instead of setInterval. The next cycle is only scheduled after the current cycle fully resolves, ensuring that a slow OPC-UA server or network delay never causes cycles to accumulate.

poll() {

await processReadRequest()   // read all groups + publish

setTimeout(poll, 500 ms)     // schedule next only after this one finishes

}

3. Module Structure

The project is split into four files. Each has a single, well-defined responsibility. All functions in index.ts are documented using standard JSDoc comments (@param, @returns, @throws) so any editor with TypeScript support will show inline hints when hovering over a function call.

| File | Type | Responsibility |
| --- | --- | --- |
| config.json | Configuration | Runtime settings: MQTT broker URL, OPC-UA endpoint, connection strategy, subscription parameters. |
| interfaces.ts | TypeScript | All shared types: TagField, TagGroup, ConnectionState enum, and the full typed AppConfig hierarchy. |
| config.ts | TypeScript | Static file-loading utilities: resolves config paths, reads tag lists, parses JSON with error handling. |
| index.ts | TypeScript | Main application: connection management, polling loop, reconnect event handlers, graceful shutdown. |

4. Configuration Reference

All runtime settings are stored in config.json at the project root. No settings are hard-coded in application logic.

MQTT Settings

| Key | Value | Description |
| --- | --- | --- |
| brokerUrl | mqtt://192.168.232.31 | MQTT broker IP address |
| mqttPort | 1883 | Standard MQTT port (unencrypted) |
| organization | magna | UNS tier 1 |
| division | iotacademy | UNS tier 2 |
| plant | conestoga | UNS tier 3 |
| area | smart | UNS tier 4 |
| line | presorter | UNS tier 5 |
| workstation | iot-group1 | UNS tier 6 |
| baseTopic | ACADEMY03 | UNS root prefix |

OPC-UA Connection Strategy

| Parameter | Value | Description |
| --- | --- | --- |
| endpoint | opc.tcp://...26543 | OPC-UA server address and port |
| applicationName | OPC-UA Reader | Client identifier sent to the server |
| initialDelay | 1000 ms | Wait before first reconnection attempt |
| maxRetry | -1 (infinite) | Retries indefinitely until successful |
| maxDelay | 30 000 ms | Maximum back-off ceiling between retries |
| randomisationFactor | 0.1 | Jitter added to avoid reconnect storms |
| endpointMustExist | false | Allows connection even if endpoint is not pre-registered |

5. Unified Namespace (UNS) Topic Structure

The UNS follows the ISA-95 enterprise hierarchy, providing a human-readable, self-describing topic path for every piece of data. All topics in this project share the same base path.

ACADEMY03/magna/iotacademy/conestoga/smart/presorter/iot-group1/cell

│         │     │          │          │     │         │          │

base      org   division   plant      area  line      station    device

Published Topics and Payload Fields

Ten messages are published per polling cycle (500 ms). Every payload includes a shared timestamp as the first field.

| Topic (relative to UNS base) | Device | Fields |
| --- | --- | --- |
| .../cell/status | Cell | initialized, running, paused, safety_enable, speed_pct, finished_part_num |
| .../cell/hbot/status | H-Bot | initialized, running, ws_violation, paused, speed_pct |
| .../cell/hbot/position | H-Bot | x, y, z |
| .../cell/hbot/torque | H-Bot | x_torque, y_torque, z_torque |

Example Payload

Topic : ACADEMY03/.../cell/hbot/position

Payload:

{

"timestamp" : "2026-06-09T14:32:01.453Z",

"x"         : 123.45,

"y"         : 67.89,

"z"         : 0.00

}

The timestamp is captured once at the top of each 500 ms cycle and injected into every group payload, allowing consumers to identify which messages were sampled together.

6. Core Functions

readGroup(session, group)

Reads all OPC-UA tags belonging to a single TagGroup in one sequential pass. Returns a field-name to value map. Individual tag failures are logged and skipped — a single bad tag does not abort the rest of the group.

| Parameter | Type | Description |
| --- | --- | --- |
| session | ClientSession | Active OPC-UA session, set to null when reconnecting |
| group | TagGroup | Defines which tags to read and their output field names |
| returns | Record<string, any> | Map of field name to raw tag value; partial on error |

publishGroup(group, fields, mqttClient, timestamp)

Serialises a tag group’s payload as JSON and publishes it to the group’s MQTT topic. The shared batch timestamp is prepended as the first field so consumers can correlate all groups published in the same cycle.

processReadRequest(mqttClient)

Executes one complete polling cycle. Captures a single ISO timestamp, then iterates every entry in TAG_GROUPS calling readGroup and publishGroup. The cycle is skipped (with a warning) if either the OPC-UA session is unavailable or the MQTT client reports offline.

connectOpc(config)

Creates the OPC-UA client, wires reconnect event handlers, and opens the initial session. Returns the client instance for use during shutdown. This function is the single point where all OPC-UA resilience logic lives.

| Event | Bridge Response |
| --- | --- |
| connection_lost | Clears activeSession; sets opcState to RECONNECTING; poll cycles skip gracefully |
| connection_reestablished | Creates a new session; sets opcState back to CONNECTED; polling resumes automatically |
| reconnection_attempt_has_failed | Logs a warning; node-opcua continues retrying per connectionStrategy |
| close | Sets opcState to DISCONNECTED; logged for visibility |

connectMqtt(config)

Connects to the MQTT broker and attaches observability event handlers (connect, reconnect, disconnect, offline, error). Uses an exponential back-off retry loop for the initial connection (up to 10 attempts, starting at 2 s, capped at 30 s). Reconnection after the initial connection is handled internally by mqtt.js.

startPolling(mqttClient) / stopPolling()

startPolling sets the running flag and launches the recursive poll() async function. stopPolling clears the flag and cancels any pending setTimeout handle. Both are safe to call multiple times.

7. Resilience & Reconnect Behaviour

OPC-UA Outage

- node-opcua detects the dropped connection and emits connection_lost.

- activeSession is set to null; opcState changes to RECONNECTING.

- Every poll cycle checks opcState and skips with a WARN log rather than throwing.

- node-opcua retries automatically using the configured back-off strategy (initialDelay: 1 s, maxDelay: 30 s, infinite retries).

- On connection_reestablished, a fresh session is opened and opcState returns to CONNECTED.

- The polling loop resumes transparently — no process restart required.

MQTT Outage

- mqtt.js detects the dropped connection and emits disconnect / offline.

- It retries internally with its own back-off strategy.

- processReadRequest checks mqttClient.connected before publishing; if false, the cycle is skipped.

- When the broker comes back, mqtt.js reconnects and emits connect; normal publishing resumes.

Simultaneous Outage

Both guards are checked independently at the start of every cycle. If either connection is down the cycle is skipped. The loop continues running so that as soon as both connections recover, publishing resumes without any external intervention.

8. Graceful Shutdown

The process listens for SIGINT (Ctrl+C) and SIGTERM (systemd stop). On either signal the following sequence runs:

- stopPolling() — cancels the pending timer and sets running = false.

- activeSession.close() — closes the OPC-UA session cleanly, freeing the server-side slot.

- opcClient.disconnect() — closes the transport connection.

- mqttClient.endAsync() — flushes any queued publishes and closes the TCP connection to the broker.

- process.exit(0) — exits with success code.

The session is closed before the client is disconnected to avoid orphaned-session warnings appearing in the OPC-UA server log.

9. Data Collection Strategy: Polling vs. Subscription

OPC-UA supports two distinct ways to read data from a PLC. This section explains why fixed-interval polling was chosen, what that choice enables, and where its limitations show up.

How Each Approach Works

|  | Fixed-Interval Polling (used here) | OPC-UA Subscription |
| --- | --- | --- |
| How it works | The client requests all tag values on a fixed timer (every 500 ms), regardless of whether any values have changed. | The client tells the server which tags to watch. The server only sends updates when a value changes (on data change, or deadband exceeded). |
| Who triggers the read | The client — it asks for data at every interval. | The server — it pushes data only when something changes. |
| Message rate | Constant and predictable. 4 groups × 2 calls/sec = always 8 messages/sec. | Variable. A quiet cell with stable values generates almost no traffic. |
| Timestamp source | One timestamp is created by the bridge at the start of each cycle and shared across all groups. | Each notification carries the server’s own timestamp for that specific change event. |

Why Polling Was Chosen

Shared Timestamp Enables Temporal Joins

The most important reason is the single shared timestamp. At the start of every 500 ms cycle, one ISO timestamp is captured and injected into every group payload published in that cycle.

{ "timestamp": "2026-06-09T14:32:01.453Z", "x": 123.45, "y": 67.89, "z": 0.00 }

{ "timestamp": "2026-06-09T14:32:01.453Z", "x_torque": 1.2, "y_torque": 0.8, "z_torque": 0.1 }

Because position and torque messages from the same cycle carry the exact same timestamp, a historian or analytics pipeline can join them on that field to reconstruct a precise snapshot of the cell at any given moment. This is not possible with a subscription model, where each tag change arrives with its own server-assigned timestamp — two values that changed a few milliseconds apart would carry different timestamps, making an exact join impossible.

Correlated Position and Torque Data

For the H-Bot, knowing that position (x, y, z) and torque (x_torque, y_torque, z_torque) came from the same instant is essential for analysing motor load relative to robot position. A subscription would deliver them as independent events timestamped by when each individual value changed, making this correlation unreliable.

Simpler Implementation

Polling requires no server-side configuration. A subscription requires setting up MonitoredItems, managing a server-side subscription object with a publish interval, keepalive counts, and lifetime counts, and handling the publish/acknowledge cycle. The polling approach is a straightforward read-on-timer loop that is easy to understand, debug, and extend.

Predictable Downstream Behaviour

Because the data rate is constant, downstream systems (MQTT brokers, historians, dashboards) receive a steady, predictable stream. There are no bursts of messages after a period of inactivity, and no need to handle “no new data” conditions in consumers.

Drawbacks of the Polling Approach

Redundant Data for Slow-Changing Tags

The most significant cost is that polling saves the same value repeatedly even when nothing has changed. Status tags like initialized, paused, and safety_enable may stay at the same value for hours or days, yet they are read and published 7,200 times per hour. This produces a large volume of identical records in any historian or database.

finished_part_num Is Especially Inefficient

The finished_part_num counter increments only when a part completes a cycle, which might happen a handful of times per minute at most. The current setup polls it 120 times per minute regardless. With a subscription, the historian would receive exactly one update per part completion.

Does Not Scale Well to Larger Cells

This cell has four tag groups and a relatively small number of tags. In a larger cell with dozens of robots, conveyors, sensors, and actuators, the constant polling of rarely-changing status tags would generate significant and largely useless traffic. At 500 ms with 40 groups, the bridge would be generating 80 messages per second, the majority carrying unchanged values.

Bandwidth and Storage Overhead

Every published message consumes MQTT broker bandwidth and historian storage. Since most status values do not change between cycles, a large percentage of the stored data is duplicated. Over weeks or months this accumulates into unnecessary storage costs and makes querying the historian slower.

Comparison Summary

| Consideration | Polling (used here) | Subscription |
| --- | --- | --- |
| Shared timestamp for joins | ✔ Yes — one timestamp per cycle, shared across all groups | ✘ No — each tag change has its own server timestamp |
| Position + torque correlation | ✔ Always in sync | ✘ May differ by milliseconds |
| Redundant data for static tags | ✘ High — status tags repeat every 500 ms | ✔ Low — only published on change |
| Traffic for a quiet cell | ✘ Constant regardless of activity | ✔ Near-zero when nothing is changing |
| Scales to large cells | ✘ Message rate grows linearly with tag count | ✔ Traffic proportional to change frequency |
| Implementation complexity | ✔ Simple read-on-timer loop | ✘ Server-side MonitoredItems, publish/ack cycle |
| Downstream predictability | ✔ Constant data rate, no bursts | ✘ Variable; consumers must handle gaps |

Design decision: Polling was chosen because the primary goal was temporal correlation — being able to join position, torque, and status data on a common timestamp to reconstruct the state of the cell at any point in time. For this cell’s scale and use case, the redundant data overhead is acceptable. For a production deployment with more devices, a hybrid approach (subscription for status tags, polling for positional data that always needs correlation) would reduce storage and bandwidth while preserving the join capability where it matters.

10. Type System (interfaces.ts)

All shared TypeScript types live in interfaces.ts. Using explicit types instead of any gives compile-time safety across the codebase and acts as self-documenting schema for the configuration file.

| Type / Enum | Category | Purpose |
| --- | --- | --- |
| TagField | interface | Pairs an OPC-UA tag string with its payload field name |
| TagGroup | interface | A topic + list of TagField entries published together |
| ConnectionState | enum | DISCONNECTED / CONNECTING / CONNECTED / RECONNECTING |
| AppConfig | interface | Top-level config.json shape |
| MqttConfig | interface | MQTT broker and UNS hierarchy settings |
| OpcConfig | interface | OPC-UA endpoint, connection strategy, identity, subscription |
| OpcConnectionStrategy | interface | Back-off parameters: initialDelay, maxRetry, maxDelay, jitter |

11. Logging

Every log line is prefixed with an ISO 8601 timestamp so that log files can be sorted and correlated without an external log-aggregation tool. Three helpers are used throughout index.ts:

| Helper | Output Stream | Usage |
| --- | --- | --- |
| log(msg) | stdout | Normal operational messages (connected, published, polling started) |
| warn(msg) | stderr | Recoverable conditions (skipped cycle, reconnecting, tag read failure) |
| logError(msg) | stderr | Unexpected errors that require attention |

[2026-06-09T14:32:01.453Z]       published: .../hbot/position -> {"timestamp":...}

[2026-06-09T14:32:03.821Z] WARN  processReadRequest: OPC-UA not connected — skipping cycle.

[2026-06-09T14:32:05.104Z]       OPC-UA connection reestablished — creating new session...