Team 1  —  H-Bot Database Schema Documentation

OPC_UA-MQTT Microservice  |  Capstone Project

1. Overview

This database stores real-time operational data for an H-Bot robot and its associated manufacturing cell. Data is inserted by the OPC_UA-MQTT database microservice, which subscribes to MQTT topics published by the OPC_UA-MQTT bridge service and writes a new row to the appropriate table on every 500 ms polling cycle.

All tables live in the public schema and follow a timestamp-based, append-only structure. This enables full time-series analysis and lets any past state of the cell be reconstructed from the historical record.

2. Layout

All tables are time-series driven and include a timestamp field. Data domains are separated so that position, robot status, torque, and cell status are stored independently. The tables can be joined on their shared timestamp to create unified snapshots of the system at any point in time.

The system follows an append-only model — rows are only ever inserted, never updated or deleted.

3. Tables

Hbot_Position

Stores the spatial position of the H-Bot end-effector in XYZ coordinates (millimetres).

| Field | Description | Data Type |
| --- | --- | --- |
| id | Auto-incrementing unique record identifier | Integer (serial) |
| timestamp | Time the OPC-UA bridge sampled this value | Timestamp |
| x_pos | Position on the X-axis (mm) | Real |
| y_pos | Position on the Y-axis (mm) | Real |
| z_pos | Position on the Z-axis (mm) | Real |

Hbot_Torque

Stores the motor torque readings for the three H-Bot axes (Newton-metres).

| Field | Description | Data Type |
| --- | --- | --- |
| id | Auto-incrementing unique record identifier | Integer (serial) |
| timestamp | Time the OPC-UA bridge sampled this value | Timestamp |
| x_torque | Torque on the X-axis motor (N·m) | Real |
| y_torque | Torque on the Y-axis motor (N·m) | Real |
| z_torque | Torque on the Z-axis motor (N·m) | Real |

Hbot_Status

Stores operational status flags and speed for the H-Bot axis controller.

| Field | Description | Data Type |
| --- | --- | --- |
| id | Auto-incrementing unique record identifier | Integer (serial) |
| timestamp | Time the OPC-UA bridge sampled this value | Timestamp |
| initialized | True once the homing / initialisation sequence completes | Boolean |
| running | True while a motion program is actively executing | Boolean |
| ws_violation | True if a workspace-limit violation is detected | Boolean |
| speed_pct | Current speed as a percentage of rated maximum (0–100) | Integer |

Cell_Status

Stores overall manufacturing cell status, including safety state and production count.

| Field | Description | Data Type |
| --- | --- | --- |
| id | Auto-incrementing unique record identifier | Integer (serial) |
| timestamp | Time the OPC-UA bridge sampled this value | Timestamp |
| initialized | True once all cell subsystems have started | Boolean |
| running | True while the cell program is executing | Boolean |
| paused | True when an operator has paused the cycle | Boolean |
| safety_enable | True when the safety circuit is closed (no E-stop active) | Boolean |
| speed_pct | Cell-wide speed override as a percentage (0–100) | Integer |
| finished_part_num | Cumulative count of successfully completed parts | Integer |

4. Data Relationships

There are no foreign-key relationships between tables. All four tables share a common timestamp field. Because all groups in a single OPC_UA-MQTT poll cycle receive the same ISO-8601 timestamp, the tables can be joined on that field to produce a unified snapshot of the system at any instant.

Example join: combine torque and position readings from the same sample cycle:

SELECT T1.id, T1."timestamp", T1.x_torque, T1.y_torque, T1.z_torque, T2.x_pos, T2.y_pos, T2.z_pos FROM public."Hbot_Torque" AS T1 LEFT JOIN public."Hbot_Position" AS T2 ON T1."timestamp" = T2."timestamp" ORDER BY T1.timestamp DESC;

5. Database Strategy: Pros & Cons

The OPC_UA-MQTT microservice inserts a new row into each table on every 500 ms poll cycle, regardless of whether any values have changed since the last cycle. This append-only, high-frequency insert model has a number of advantages and trade-offs that are worth understanding, especially when considering how this design might scale beyond a capstone environment.

|  | Consideration |
| --- | --- |
| PRO | Complete historical record — every row is timestamped and nothing is ever overwritten, so the exact state of the cell can be reconstructed at any point in the past. |
| PRO | Consistent snapshots — because all four tables share the same timestamp per cycle, they can always be joined to produce a coherent, point-in-time view of the whole system. |
| PRO | Simple implementation — unconditional INSERTs are easier to write, test, and reason about than conditional UPDATE-or-INSERT (UPSERT) logic. |
| PRO | Good fit for analytics and dashboards — time-series tools (e.g., Grafana, TimescaleDB) are optimised for append-only data. Trend analysis, anomaly detection, and event replay are all straightforward. |
| PRO | No race conditions from concurrent writes — each insert is independent; there is no shared row that two writers could corrupt by updating simultaneously. |
| PRO | Audit trail is implicit — the table itself acts as a log. For a capstone system this eliminates the need for a separate logging layer. |
| CON | Redundant data volume — status values typically stay the same across many cycles. Most rows in Hbot_Status and Cell_Status contain identical values to the previous row, wasting storage on unchanged state. |
| CON | Storage grows without bound — at 500 ms intervals across four tables, the database accumulates roughly 7,200 rows per minute. On a larger production line with more robots and tags, this multiplies quickly and requires a retention / archiving policy to remain manageable. |
| CON | Query performance degrades over time — without table partitioning or regular archiving, queries that scan large date ranges will slow down as row counts grow. |
| CON | An UPDATE-based alternative for status tables would reduce storage and always expose the current state directly, but at the cost of losing all history — you would no longer be able to see how long the cell was paused, when a workspace violation occurred, or how speed changed over a shift. |
| CON | Not well suited to a high-tag-count production environment — a real factory line may have hundreds of PLCs and thousands of tags. This design would need significant rework (partitioning, TimescaleDB hypertables, or a dedicated time-series store) to remain performant at that scale. |

For this capstone application the append-only strategy is appropriate. The data volume is small, the full history is useful for demonstrations and analysis, and the simplicity of the implementation reduces the chance of bugs. The trade-offs would become significant in a full production deployment and would require a purpose-built data retention strategy.

6. Important Notes

- Table names are case-sensitive in PostgreSQL and must be wrapped in double-quotes in all queries (e.g. "Hbot_Torque").

- The timestamp field stores values without a time zone (local server time). Ensure all services writing to the database are on the same time zone to avoid skew when joining tables.

- The id column is a PostgreSQL serial (auto-incrementing integer). It is never sent over MQTT; it is generated by the database on insert.

- Rows are never updated or deleted by the microservice. Any maintenance queries (e.g., pruning old data) should be run manually or via a scheduled job.

- Microservice must have the proper env variables set up in order to run.